GENERAL

The DMA handles the bulk of memory transfers in Playstation2. There are 10 DMA channels

handling the memory transfers to and from the different targets. The operation of each channel

is similar. The DMA reads 1 qword from the source memory pool and transfers it to the target.

It then updates the internal counters and loops ,if necessary.

DMA CHANNELS

Channel 0 - VIF0

Channel 1 - VIF1

Channel 2 - GIF

Channel 3 - from IPU

Channel 4 - to IPU

Channel 5 - SIF0

Channel 6 - SIF1

Channel 7 - SIF2

Channel 8 - from SPR

Channel 9 - to SPR

REGISTERS

Each DMA channel has the following registers:

TADR	Tag address			  , Address of DMA Tag for current DMA packet

MADR	Memory address		  , Address of DMA packet quadraword accessed

QWC	Quadra-word counter 	  , Qwords remaining in DMA packet

CHCR  Channel Control register  , Control register. Handles :

Start \Busy ,

direction   ,

mode	    ,

TIE 	    ,

TTE

ASR0	Assembler Reserved 0

ASR1	Assembler Reserved 1

CHCR BIT ENCODING

_____________________________________________________________________________________________

|31|										  | 08  |07 |06 |05|04|03|02|01|00|

|  |										  |start|TIE|TTE|mode |Unkwn| dir |

|  |										  |     |   |   |     |	    |     |

start Start operation     , DMA operation switch  0 No operation

1 Start operation

TIE  				  , Turned to 0

TTE   Tag transfer enable , Whether the first part of DMA data should be packed in with the

qword containing the DMA tag

mode  Mode selector	  , Control the DMA transfer mode   00 Chain

01

10

11

Unkwn Unknown		  , This element is set to 01 for VIF DMA transfers

dir   Direction		  , DMA transfer direction  00 to memory

01 from memory

Bit 31 could be busy bit (for when DMA is within Packet ,not on a tag)