LOWER VU INSTRUCTION LAYOUT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	OPCODE	   | x| y| z| w|      rt      |      rs      |      rd  	| SPECIAL | bc  |

|			   | t.e | s.e |		      |		   |			|	    |	    |

ft.e and fs.e : 00 x

01 y

10 z

11 w

LOWER VU INSTRUCTION ENCODING

Branch														  B

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    			  00|

|	    B		   | x| y| z| w|       0      |       0      |		    offset		    |

|	 0100000	   | 0| 0| 0| 0|	   00000    |	00000    |					    |

Format:  B offset

Purpose:

To unconditionally branch to a VU instruction address

Description: PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Branch and link												      BAL

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    			  00|

|	   BAL	   | x| y| z| w|      It      |       0      |		    offset		    |

|	 0100001	   | 0| 0| 0| 0|			|	00000    |					    |

Format:  BAL It, offset

Purpose:

To unconditionally branch to a VU instruction address and link to an IR

Description: It <- PC + 16

PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Divide														DIV

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      ft      |      fs      |     DIV  	| SPECIAL | bc  |

|  	 1000000	   | t.e | s.e |		      |		   |    01110  	|   1111  | 00  |

Format:  DIV Q,ft.e,fs.e

Purpose:

To divide a field of a VU FPR with a field of a VU FPR and store the result in the Q

register

Description: Q <- ft.e/fs.e

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

EFU Arctangent												    EATAN

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    EATAN  	| SPECIAL | bc  |

|  	 1000000	   | 0| 0| s.e |	  00000     |		   |    11111  	|   1111  | 01  |

Format:  EATAN P, fs.e

Purpose:

To calculate the arctangent of a single field in a VU FPR and store it in the P register

Description: P <- arctangent(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Arctangent xy											  EATANxy

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    EATAN  	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 0| 0|	  00000     |		   |    11101  	|   1111  | 00  |

Format:  EATANxy P, fs

Purpose:

To calculate the arctangent of two fields in a VU FPR and store it in the P register

Description: P <- arctangent(fs.x, fs.y)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Arctangent xz											  EATANxz

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    EATAN  	| SPECIAL | bc  |

|  	 1000000	   | 1| 0| 1| 0|	  00000     |		   |    11101  	|   1111  | 01  |

Format:  EATANxy P, fs

Purpose:

To calculate the arctangent of two fields in a VU FPR and store it in the P register

Description: P <- arctangent(fs.x, fs.z)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Exponent												     EEXP

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    EEXP  	| SPECIAL | bc  |

|  	 1000000	   | 0| 0| s.e |	  00000     |		   |    11111  	|   1111  | 10  |

Format:  EEXP P, fs.e

Purpose:

To calculate the exponent of a single field of a VU FPR and store it in the P register

Description: P <- exponent(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Length													    ELENG

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ELENG 	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 1| 0|	  00000     |		   |    11100  	|   1111  | 10  |

Format:  ELENG P, fs

Purpose:

To calculate the length for the xyz fields of a VU FPR and store it in the P register

Description: P <- length(fs.xyz)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Reciprocal root											    ERCPR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ERCPR 	| SPECIAL | bc  |

|  	 1000000	   | 0| 0| s.e |	  00000     |		   |    11110  	|   1111  | 10  |

Format:  ERCPR P, fs.e

Purpose:

To calculate the reciprocal root for a single field of a VU FPR and store it in the P

register

Description: P <- rec.root(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Reverse length											   ERLENG

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ERLENG	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 1| 0|	  00000     |		   |    11100  	|   1111  | 11  |

Format:  ERLENG P, fs.xyz

Purpose:

To calculate the reverse length for the xyz fields of a VU FPR and store it in the P

register

Description: P <- rev.len(fs.xyz)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Reverse square root add									  	   ERSADD

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ERSADD	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 1| 0|	  00000     |		   |    11100  	|   1111  | 01  |

Format:  ERSADD P,fs

Purpose:

To calculate the reverse square root for the sum of the xyz fields in a VU FPR and

store it in the P register

Description: P <- rev.sq.root(fs.x + fs.y + fs.z)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Reverse square root										   ERSQRT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ERSQRT	| SPECIAL | bc  |

|  	 1000000	   | 0  0| s.e |	  00000     |		   |    11110  	|   1111  | 01  |

Format:  ERSQRT P, fs.e

Purpose:

To calculate the reverse square root of a single field of a VU FPR and store it in the P

register

Description: P <- rev.sq.root(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Square root add											    ESADD

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ESADD	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 1| 0|	  00000     |		   |    11100  	|   1111  | 00  |

Format:  ESADD P, fs.xyz

Purpose:

To calculate the square root of the sum of the xyz fields of a VU FPR and store it in

the P register

Description: P <- sq.root(fs.x + fs.y + fs.z)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Sine													     ESIN

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ESIN	| SPECIAL | bc  |

|  	 1000000	   | 0| 0| s.e |	  00000     |		   |    11111  	|   1111  | 00  |

Format:  ESIN P, fs.e

Purpose:

To calculate the sine of a single field of VU FPR and store it in the P register

Description: P <- sine(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Square root												    ESQRT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ESQRT	| SPECIAL | bc  |

|  	 1000000	   | 0| 0| s.e |	  00000     |		   |    11110  	|   1111  | 00  |

Format:  ESQRT P, fs.e

Purpose:

To calculate the square root of a single field of VU FPR and store it in the P register

Description: P <- sq.root(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

EFU Sum		 											     ESUM

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05     02|01|00|

|	 SPECIAL 	   | x| y| z| w|      0       |      fs      |    ESUM	| SPECIAL | bc  |

|  	 1000000	   | 1| 1| 1| 1|	  00000     |		   |    11101  	|   1111  | 10  |

Format:  ESUM P,fs

Purpose:

To calculate the sum of all the fields of a VU FPR and store it in the P register

Description: P <- fs.x + fs.y + fs.z + fs.w

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

Clipping AND												    FCAND

_____________________________________________________________________________________________

|31			 25|24|23											  00|

|	  FCAND  	   | x|					immediate					    |

|  	 0010010	   | 0| 											    |

Format:  FCAND VI01,imm24

Purpose:

To AND the clipping flag with the 24 bit immediate and move the result to the VI01

register

Description: VI01 <- clipping flag AND imm24

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Clipping equality											     FCEQ

_____________________________________________________________________________________________

|31			 25|24|23											  00|

|	  FCEQ	   | x|					immediate					    |

|  	 0010000	   | 0| 											    |

Format:  FCEQ VI01,imm24

Purpose:

To compare the clipping flag with the 24 bit immediate and move the clipping flag to the

VI01 register on equality

Description: clipping flag == imm24  :  VI01 <- clipping flag

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Clipping transfer											    FCGET

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FCGET 	   | x| y| z| w|	    It      |	 0	   |			0		    |

|  	 0011100	   | 0| 0| 0| 0|			|    00000	   |         00000000000          |

Format:  FCGET It

Purpose:

To move the clipping flag to an IR

Description: It <- clipping flag

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Clipping OR											     	     FCOR

_____________________________________________________________________________________________

|31			 25|24|23											  00|

|	  FCOR	   | x|					immediate					    |

|  	 0010011	   | 0| 											    |

Format:  FCOR VI01,imm24

Purpose:

To OR the clipping flag with the 24 bit immediate and store the result in the VI01

register

Description: VI01 <- clipping flag OR imm24

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Clipping set											     	    FCSET

_____________________________________________________________________________________________

|31			 25|24|23											  00|

|	  FCSET 	   | x|					immediate					    |

|  	 0010001	   | 0| 											    |

Format:  FCSET imm24

Purpose:

To set the clipping flag to the value of the 24 bit immediate

Description: clipping flag == imm24

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

MAC AND													    FMAND

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FMAND 	   | x| y| z| w|	    It      |	 Is	   |			0		    |

|  	 0011010	   | 0| 0| 0| 0|			|		   |         00000000000          |

Format:  FMAND It,Is

Purpose:

To AND the MAC flag with an IR and store the result in an IR

Description: It <- MAC AND Is

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

MAC equality												     FMEQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FMEQ 	   | x| y| z| w|	    It      |	 Is	   |			0		    |

|  	 0011000	   | 0| 0| 0| 0|			|		   |         00000000000          |

Format:  FMEQ It,Is

Purpose:

To compare the MAC flag with the value of an IR and move the MAC flag to an IR on

equality

Description: MAC == Is  :  It <- MAC

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

MAC OR													     FMOR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FMOR 	   | x| y| z| w|	    It      |	 Is	   |			0		    |

|  	 0011011	   | 0| 0| 0| 0|			|		   |         00000000000          |

Format:  FMOR It,Is

Purpose:

To OR the MAC flag with an IR and store the result in an IR

Description: It <- MAC OR Is

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Status AND													    FSAND

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FSAND	   | x| y| z| w|	    It      |	  0 	   |		  immediate		    |

|  	 0010110	   | 0| 0| 0| *|			|	00000    |	  12th bit is in dest.w	    |

Format:  FSAND It,imm12

Purpose:

To AND the status flag with the 12 bit immediate and store the result in an IR

Description: It <- STATUS AND imm12

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Status equality												     FSEQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FSEQ	   | x| y| z| w|	    It      |	  0 	   |		  immediate		    |

|  	 0010100	   | 0| 0| 0| *|			|	00000    |	  12th bit is in dest.w	    |

Format:  FSEQ It, imm12

Purpose:

To compare the status flag with the 12 bit immediate and move the status flag to an IR

on equality

Description: STATUS == imm12  :  It <- STATUS

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Status OR												    	     FSOR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FSOR	   | x| y| z| w|	    It      |	  0 	   |		  immediate		    |

|  	 0010111	   | 0| 0| 0| *|			|	00000    |	  12th bit is in dest.w	    |

Format:  FSOR It, imm12

Purpose:

To OR the status flag with the 12 bit immediate and store the result in an IR

Description: It <- STATUS OR imm12

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Status set												    	    FSSET

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  FSSET	   | x| y| z| w|      0       |	  0 	   |		  immediate		    |

|  	 0010101	   | 0| 0| 0| *|    00000     |	00000    |	  12th bit is in dest.w	    |

Format:  FSSET imm12

Purpose:

To set the status	flag to the value of the 12bit immediate

Description: STATUS <- imm12

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Integer add											    	     IADD

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 Is	   |	    Id      |	 IADD	    |

|  	 1000000	   | 0| 0| 0| 0|			|		   |			|     110000    |

Format:  IADD Id, Is, It

Purpose:

To add two IRs and store the result in an IR

Description: Id <- Is + It

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Integer add immediate 										    	    IADDI

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 Is	   |	    imm     |	IADDI	    |

|  	 1000000	   | 0| 0| 0| 0|			|		   |			|     110010    |

Format:  IADDI It, Is, imm5

Purpose:

To add an IR with a signed 5 bit immediate and store the result in an IR

Description: It <- Is + imm5

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

The immediate's value ranges from -16 to 15

Integer add immediate unsigned								    	   IADDIU

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	 IADDIU	   | x| y| z| w|	    It      |	 Is	   |		  immediate		    |

|  	 0001000	   | *  *  *  *|			|		   | the upper 4 bits are in dest |

Format:  IADDIU It, Is, imm15

Purpose:

To add an IR with an unsigned 15 bit immediate and store the result in an IR

Description: It <- Is + imm15

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The immediate's value ranges from 0 to 32767

Integer AND											    	     IAND

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 Is	   |	    Id      |	 IAND	    |

|  	 1000000	   | 0| 0| 0| 0|			|		   |			|     110100    |

Format:  IAND Id, Is, It

Purpose:

To AND two IRs and store the result in an IR

Description: Id <- Is AND It

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Integer branch on equal									    	     IBEQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  IBEQ	   | x| y| z| w|	    It      |	 Is	   |		     offset		    |

|  	 0101000	   | 0| 0| 0| 0|			|		   |					    |

Format:  IBEQ It, Is, offset

Purpose:

To branch to a VU instruction memory offset if two IRs are equal

Description: It == Is  :  PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Integer branch on greater or equal to zero						    	    IBGEZ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  IBGEZ	   | x| y| z| w|	    0       |	 Is	   |		     offset		    |

|  	 0101111	   | 0| 0| 0| 0|	  00000     |		   |					    |

Format:  IBGEZ Is, offset

Purpose:

To branch to a VU instruction memory offset if the value in an IR is greater or equal to

zero

Description: Is >=0  :  PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Integer branch on greater than zero						    	 	    IBGTZ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  IBGTZ	   | x| y| z| w|	    0       |	 Is	   |		     offset		    |

|  	 0101101	   | 0| 0| 0| 0|	  00000     |		   |					    |

Format:  IBGTZ Is, offset

Purpose:

To branch to a VU instruction memory offset if the value an IR is greater than zero

Description: Is >0  :  PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Integer branch on less or equal to zero						    	 	    IBLEZ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  IBLEZ	   | x| y| z| w|	    0       |	 Is	   |		     offset		    |

|  	 0101110	   | 0| 0| 0| 0|	  00000     |		   |					    |

Format:  IBLEZ Is, offset

Purpose:

To branch to a VU instruction memory offset if the value in an IR is less or equal to

zero

Description: Is <=0  :  PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Integer branch on not equal								    	 	     IBNE

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  IBNE	   | x| y| z| w|	    It      |	 Is	   |		     offset		    |

|  	 0101001	   | 0| 0| 0| 0|			|		   |					    |

Format:  IBNE It, Is, offset

Purpose:

To branch to a VU instruction memory offset if two IRs are not equal

Description: Is =| It  :  PC <- PC + offset*8 + 8

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023 giving it an effective address range of 0 to 8184 in

8 byte steps

Integer load word								    	 	    		ILW

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  ILW		   | x| y| z| w|	    It      |	base	   |		     offset		    |

|  	 0000100	   |    dest   |			|		   |					    |

Format:  ILW.dest It, offset(base)

Purpose:

To load a word from the VU data cache to an IR

Description: It <- offset(base).dest

Restrictions:

The .dest field can hold only one destination

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023

Integer load word register							    	 	    	     ILWR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	base	   |    ILWR	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01111  	|	111110    |

Format:  ILWR.dest It, base

Purpose:

To load a word from the VU data cache to an IR

Description: It <- [base].dest

Restrictions:

The .dest field can hold only one destination

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Integer OR											    	     		IOR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 Is	   |	    Id      |	 IOR	    |

|  	 1000000	   | 0| 0| 0| 0|			|		   |			|     110011    |

Format:  IOR Id, Is, It

Purpose:

To OR two IRs and store the result in an IR

Description: Id <- Is OR It

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Integer substract									    	     	     ISUB

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 Is	   |	    Id      |	 ISUB	    |

|  	 1000000	   | 0| 0| 0| 0|			|		   |			|     110001    |

Format:  ISUB Id, Is, It

Purpose:

To substract two IRs and store the result in an IR

Description: Id <- Is - It

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Integer substract immediate unsigned								   ISUBIU

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	 ISUBIU	   | x| y| z| w|	    It      |	 Is	   |		  immediate		    |

|  	 0001001	   | *  *  *  *|			|		   | the upper 4 bits are in dest |

Format:  ISUBIU It, Is, imm15

Purpose:

To substract an unsigned 15 bit immediate from an IR and store the result in an IR

Description: It <- Is - imm15

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The immediate's value ranges from 0 to 32767

Integer store word								    	 	    		ISW

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  ISW		   | x| y| z| w|	    It      |	base	   |		     offset		    |

|  	 0000101	   |    dest   |			|		   |					    |

Format:  ISW.dest It, offset(base)

Purpose:

To store an IR in the VU data cache

Description: offset(base).dest <- It

Restrictions:

The .dest field can hold only one destination

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023

Integer store word register							    	 	    	     ISWR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	base	   |    ISWR	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01111  	|	111111    |

Format:  ISWR.dest It, base

Purpose:

To store an IR in the VU data cache

Description: [base].dest <- It

Restrictions:

The .dest field can hold only one destination

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023

Jump and link register											     JALR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	  JALR	   | x| y| z| w|	    It      |	 Is	   |			0		    |

|  	 0100101	   | 0| 0| 0| 0|			|		   |		 00000000000	    |

Format:  JALR It, Is

Purpose:

To branch to the VU instruction memory offset of an IR and link to an IR

Description: It <- PC + 8

PC <- Is

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Jump register											    		 JR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	   JR 	   | x| y| z| w|	    0       |	 Is 	   |			0		    |

|  	 0100100	   | 0| 0| 0| 0|	  00000	|		   |		 00000000000	    |

Format:  JALR Is

Purpose:

To branch to the VU instruction memory offset of an IR

Description: PC <- Is

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

Load quadraword								    	 	    			 LQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	   LQ		   | x| y| z| w|	    ft      |	base	   |		     offset		    |

|  	 0000000	   |    dest   |			|		   |					    |

Format:  LQ.dest ft, offset(base)

Purpose:

To load up to 4 fields of a VU FPR with the contents of a quadraword in the VU data cache

Description: ft.dest <- offset(base)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023

Load quadraword pre decrement					    	 	    			LQD

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	base	   |     LQD	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01101  	|	111110    |

Format:  LQD.dest ft, --base

Purpose:

To load up to 4 fields of a VU FPR with the contents of a quadraword in the VU data

memory with address pre decrement

Description: base    <- base - 16

ft.dest <- [base]

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Load quadraword post increment					    	 	    			LQI

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	base	   |     LQI	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01101  	|	111100    |

Format:  LQI.dest ft, base++

Purpose:

To load up to 4 fields of a VU FPR with the contents of a quadraword in the VU data

memory with address post increment

Description: ft.dest <- [base]

base    <- base + 16

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Move from integer register					    	 	    			     MFIR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	 Is 	   |    MFIR	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01111  	|	111101    |

Format:  MFIR.dest ft, Is

Purpose:

To load a field of a VU FPR with the value of an IR

Description: ft.dest <- Is

Restrictions:

The .dest field can hold only one destination

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Move floating point registers		    	 	    					     MOVE

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	 fs  	   |    MOVE	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01100  	|	111100    |

Format:  MOVE.dest ft, fs

Purpose:

To move up to 4 fields of a VU FPR to matching fields of a VU FPR

Description: .dest includes .x  :  ft.x <- fs.x

.dest includes .y  :  ft.y <- fs.y

.dest includes .z  :  ft.z <- fs.z

.dest includes .w  :  ft.w <- fs.w

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

This instruction also serves as the NOP instruction for ft and fs fields set to 0

Move and rotate per word			    	 	    					     MR32

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	 fs  	   |    MR32	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01100  	|	111101    |

Format:  MOVE.dest ft, fs

Purpose:

To move up to 4 fields of a VU FPR to rotated fields of a VU FPR

Description: .dest includes .x  :  ft.y <- ft.x

.dest includes .y  :  ft.z <- ft.y

.dest includes .z  :  ft.w <- fs.z

.dest includes .w  :  ft.x <- fs.w

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Move to integer register					    	 	    			     MTIR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 fs 	   |    MTIR	|	SPECIAL   |

|  	 1000000	   | 0| 0| fs.e|			|		   |    01111  	|	111100    |

Format:  MTIR.e It, fs.e

Purpose:

To move a field of a VU FPR to an IR

Description: It <- fs.e

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

R move					    	 	    			     			     RGET

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	  0 	   |    RGET	|	SPECIAL   |

|  	 1000000	   |    dest   |			|	00000	   |    10000  	|	111101    |

Format:  RGET.dest ft, R

Purpose:

To load up to 4 fields of a VU FPR with the R register

Description: fs.dest <- R

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

R initialise				    	 	    			     			    RINIT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    0       |	 fs	   |    RINIT	|	SPECIAL   |

|  	 1000000	   | 0| 0| fs.e|	  00000	|		   |    10000  	|	111110    |

Format:  RINIT.e R, fs.e

Purpose:

To initialise the random generator using a field of a VU FPR as matrix

Description: R <- Random(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

R next					    	 	    			     			    RNEXT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	  0 	   |    RNEXT	|	SPECIAL   |

|  	 1000000	   |    dest   |			|	00000	   |    10000  	|	111100    |

Format:  RNEXT.dest ft, R

Purpose:

To load up to 4 fields of a VU FPR with the next random number from the R register

Description: ft.dest <- R

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Reverse square root			    	 	    			     			    RSQRT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	 fs	   |    RSQRT	|	SPECIAL   |

|  	 1000000	   | ft.e| fs.e|			|		   |    01110  	|	111110    |

Format:  RSQRT Q, ft.e, fs.e

Purpose:

To calculate ft.e/sqrt(fs.e)

Description: Q <- ft.e/sqrt(fs.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

R XOR				    	 	    			     				     RXOR

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    0       |	 fs	   |    RXOR	|	SPECIAL   |

|  	 1000000	   | 0| 0| fs.e|	  00000	|		   |    10000  	|	111111    |

Format:  RXOR.e R, fs.e

Purpose:

To XOR the value in the R register and a field of a VU FPR and store the result in the

R register

Description: R <- R XOR fs.e

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Store quadraword								    	 	    			 SQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10				  00|

|	   SQ		   | x| y| z| w|	    ft      |	base	   |		     offset		    |

|  	 0000001	   |    dest   |			|		   |					    |

Format:  SQ.dest ft, offset(base)

Purpose:

To store up to 4 fields of a VU FPR in a quadraword in the VU data cache

Description: offset(base) <- fs.dest

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				Yes				Yes

Programming notes:

The offset ranges from 0 to 1023

Store quadraword pre decrement					    	 	    			SQD

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	   base	|	 ft	   |     SQD	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01101  	|	111111    |

Format:  SQD.dest ft, --base

Purpose:

To store up to 4 fields of a VU FPR in a quadraword in the VU data memory with address

pre decrement

Description: base   <- base - 16

[base] <- ft.dest

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Store quadraword post increment					    	 	    			SQI

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	   base	|	 ft	   |     SQI	|	SPECIAL   |

|  	 1000000	   |    dest   |			|		   |    01101  	|	111101    |

Format:  SQI.dest ft, base++

Purpose:

To store up to 4 fields of a VU FPR in a quadraword in the VU data memory with address

pre decrement

Description: [base] <- ft.dest

base   <- base + 16

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Square root			    	 	    			     				     SQRT

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    ft      |	 0 	   |    SQRT	|	SPECIAL   |

|  	 1000000	   | ft.e| 0| 0|			|    00000	   |    01110  	|	111101    |

Format:  SQRT Q, ft.e

Purpose:

To calculate the square root of the value in a single field of a VU FPR

Description: Q <- sqrt(ft.e)

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Wait P operation			    	 	    			     				    WAITP

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    0       |	 0 	   |    WAITP	|	SPECIAL   |

|  	 1000000	   | 0| 0| 0| 0|	  00000	|    00000	   |    11110  	|	111111    |

Format:  WAITP

Purpose:

To postpone VU operation untill the instruction that accesses the P register has exited

the pipeline

Description:

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

Wait Q operation			    	 	    			     				    WAITQ

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    0       |	 0 	   |    WAITQ	|	SPECIAL   |

|  	 1000000	   | 0| 0| 0| 0|	  00000	|    00000	   |    01110  	|	111111    |

Format:  WAITQ

Purpose:

To postpone VU operation untill the instruction that accesses the Q register has exited

the pipeline

Description:

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

Yes				Yes				Yes

Programming notes:

Initialise GIF path		    	 	    			     				   XGKICK

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    0       |	 Is	   |    XGKICK	|	SPECIAL   |

|  	 1000000	   | 0| 0| 0| 0|	  00000	|		   |    11011  	|	111100    |

Format:  XGKICK Is

Purpose:

To initialise data transfer to the GIF from the VU data memory address refered to by an

IR

Description: GIF <- GIF package [Is]

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

GIF path handling		    	 	    			     				    XITOP

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 0	   |    XITOP	|	SPECIAL   |

|  	 1000000	   | 0| 0| 0| 0|			|    00000	   |    11010  	|	111101    |

Format:  XITOP It

Purpose:

To control the VU1 path to the GIF

Description:

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

GIF path handling		    	 	    			     				     XTOP

_____________________________________________________________________________________________

|31			 25|24|23|22|21|20	    16|15		 11|10	    06|05		  00|

|	 SPECIAL	   | x| y| z| w|	    It      |	 0	   |    XTOP	|	SPECIAL   |

|  	 1000000	   | 0| 0| 0| 0|			|    00000	   |    11010  	|	111100    |

Format:  XTOP It

Purpose:

To control the VU1 path to the GIF

Description:

Restrictions:

None

Operation:

Exceptions:

VU Application:	VU0 (co-processor)	VU0 (microprocessor)		VU1

No				No				Yes

Programming notes:

LEGEND

EFU	Elementary Processor Unit

FPR	Floating point register

IR	Integer register

VU 	Vector Unit

VU0	Vector Unit 0

VU1	Vector Unit 1

bc	broadcast field

dest	destination fields

.e	single field selector

fd	floating point destination register

ft	floating point target register

fs	floating point source register

Id	integer destination register

It	integer target register

Is	integer source register

imm	immediate

Last updated

21/9/2001